Buckeye International, Inc.

2700 Wagner Place Maryland Heights, MO 63043 314/291-1900

Material Safety Data Sheet

N. F. P. A.

4 = Extreme 3 = High 2 = Moderate 1 = Slight

0 = Insignificant

| "-• HEALTH | 3 |
| --- | --- |
| ^- FIRE | 0 |
| ^^_ REACTIVITY | 0 |

24 Hour Medical Emergency Telephone Number:   1-800-303-0441. Outside North America call: 651-632-8956.

24 Hour Transportation Emergency Telephone Number:   1-800-535-5053. Outside North America call: 352-323-3500.

| SECTION I - IDENTIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| PRODUCT NAME BUCKEYE |  | SANICARE LEMON QUAT |  | DATE PREPARED April 1,2005 |
| CHEMICAL FAMILY Cleaner Disinfectant, Water Based |  |  |  | CODE 5076 |
| PROPER D.O.T. shipping name Disinfectant, |  | Liquid, NOIBN |  |  |
| D.O.T. HAZARD classification None |  |  |  |  |
| SECTION II - INGREDIENTS AND IDENTITY INFORMATION |  |  |  |  |
| % By WGT | MATERIAL |  | PEL T.L.V. | C.A.S. NO. |
| 4.23 | Blend of Didecyl Dimethyl and n-Alkyl |  | NE NE | 7173-51-5 |
|  | Dimethyl Benzyl Ammoniun Chlorides |  |  | 8001-54-5 |
| >90.77 | Soft Water |  | NE NE | 7732-18-5 |
| <4.0 | Octyl Dimethyl Amine Oxide |  | NE NE | 2605-78-9 |
| <1.0 | Perfume, coloring and additives less than 1 % |  | NA NA | NA |
| ITEMS MARKED * ARE SARA TITLE III SEC 313 REPORTABLES. ALL INGREDIENTS ARE ON TSCA INVENTORY. |  |  |  |  |
| SECTION III - PHYSICAL DATA |  |  |  |  |
| BOILING POINT °F |  | 212°F | pH (CONC.) | 7.6 ± 0.2 |
| SOLUBILITY IN WATER |  | Infinite | pH (USE DILUTION) 1:64 | 7.0 ±0.2 |
| % VOLATILE BY WEIGHT |  | 93.0 | EVAPORATION RATE (Water=l) | 1.0 |
| SPECIFIC GRAVITY |  | 1.00 | PRODUCT FORM | Liquid |
| APPEARANCE AND ODOR Lemon, Clear Yellow Solution |  |  |  |  |
| SECTION IV - FIRE AND EXPLOSION DATA |  |  |  |  |
| FLASH POINT(Test Method) Tag Closed Cup: None! |  |  | FLAMMABLE LIMITS |  |
| EXTINGUISHING MEDIA |  | NA | UPPER LIMIT: NA | LOWER LIMIT: NA |
| SPECIAL FIRE FIGHTING PROCEDURES |  | None |  |  |
| UNUSUAL FIRE AND EXPLOSION HAZARDS |  | ¦ Products of combustion. |  |  |
| Oxides of carbon and nitrogen. |  |  |  |  |
| SECTION V - REACTIVITY DATA |  |  |  |  |
| STABILITY Stable |  |  |  |  |
| CONDITIONS TO AVOID None known. |  |  |  |  |
| INCOMPATIBILITY Do not mix with chlorine bleach or anionic detergents. |  |  |  |  |
| HAZARDOUS DECOMPOSITION PRODUCTS |  | ¦ None known. |  |  |
| HAZARDOUS POLYMERIZATION |  | ¦ Will not occur. |  |  |

SECTION VI - HEALTH HAZARD DATA

ROUTE(S) OF ENTRY:	INHALATION?    Yes	SKIN? Yes	INGESTION?   No

HEALTH HAZARDS (Acute and Chronic)

Corrosive - causes irreversible eye damage. Causes skin irritation. Harmful if inhaled.

CARCINOGENICITY:	NTP?   No	IARC MONOGRAPHS?     No	OSHA REGULATED?    No

SIGNS AND SYMPTOMS OF OVEREXPOSURE

For Eyes: Redness or burning sensation.

For Skin: Redness of skin or a warming sensation.

MEDICAL CONDITIONS

GENERALLY AGGRAVATED BY EXPOSURE

None known.

EMERGENCY AND FIRST AID PROCEDURES

Call a poison control center or doctor immediately for treatment advice. For Eyes and Skin: In case of
contact, immediately flush eyes or skin with plenty of water for 15 - 20 minutes. For Inhalation: Move person
to fresh air. If person is not breathing, call 911 or an ambulance and then give artificial respiration. For Ingestion:
Give two large glasses of water. DO NOT induce vomiting. Never give anything by mouth to an unconscious person.
Note to Physician: Probable mucosal damage may contraindicate the use of gastric lavage.

SECTION VII - SPILL OR LEAK PROCEDURES

SPILL RESPONSE

Pick up with mop, wet/dry vac or absorbent material. Rinse area with clear water and allow floor to dry
before allowing traffic.

WASTE DISPOSAL

METHOD   Flush to sanitary sewer or send to sanitary landfill, following local, state and federal laws.

SECTION VIII - SPECIAL PROTECTION INFORMATION

EYE PROTECTION                                                                                      VENTILATION
Wear safety glasses or chemical splash goggles.	Normal room ventilation.

SKIN                                      Rubber gloves or other                           RESPIRATORY PROTECTION
PROTECTION	impervious gloves.	Avoid breathing spray mist.

OTHER

PROTECTION	Wash thoroughly with soap and water after handling.

Remove contaminated clothing and wash clothing before reuse.

SECTION IX - SPECIAL PRECAUTIONS

PRECAUTIONS IN	KEEP OUT OF REACH OF CHILDREN!

HANDLING AND STORAGE	Rinse container before discarding.

Keep container closed when not in use.
	Store at room temperature.

OTHER	This product is not regulated under CERCLA or RCRA.

PRECAUTIONS	If more than 10,000 Ibs. are stored in a single day, product may require

NA = Not Applicable	reporting under SARA Title III, Section 311/312 as an immediate health hazard.

NE = Not Established

PREPARED BY;	Mark Gindling, Director of Research

Disclaimer of Liability

As the conditions or methods of use are beyond our control, we do not assume any responsibility and expressly disclaim any liability for any use of the material. Information contained herein is believed to be true and accurate but all statements or suggestions are made without any warranty, expressed or implied, regarding accuracy of the information, the hazards connected with the use of the material or the results to be obtained from the use thereof. Conforms to OSHA 174, Sept 1985.