davies-young company

2700 Wagner Place, Maryland Heights, Missouri 63043

Material Safety Data

N.F.P.A.

4 = Extreme

3 = High

2 = Moderate

1 = Slight

0 = Insignificant

U   ID    HEALTH

Emergency Telephone: 314/291-1900 ( 8 a.m. -5 p.m.) 314/344-2077 (5 p.m. -8a.m.)

| SECTION 1 - IDENTIFICA TION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PRODUCT NAME BUCKEYE SPARKLE |  | DATE PREPARED April 13, 1989 |  |  |  |
| CHEMICAL FAMILY Acid Cleaner |  | CODE 5260 |  |  |  |
| -*ROPFR P) O T shipping name Compound, Cleaning, Liquid, (Containing Phosphoric Acid) NA 1760 |  |  |  |  |  |
| D.O.T. HAZARD _.....- classification Corrosive Material |  |  |  |  |  |
| SECTION II - HAZARDOUS INGREDIENTS (OSHA 29 CFR 1910. 1200) |  |  |  |  |  |
| % BY WEIGHT | MATERIAL |  | T.L.V. |  | C.A.S. NO. |
| 48.5 | 75% Phosphoric Acid* |  | 1 mg/rn3 |  | 7664-38-2 |
| SECTION II-B - NON-HAZARDOUS INGREDIENTS |  |  |  |  |  |
| % BY WEIGHT RANGE | MATERIAL |  |  |  | C.A.S. NO. |
| 0.5-2.0 | Perfume, Coloring, Wetting Agent |  |  |  | - |
| 50.0-51.0 | Water |  |  |  | - |
| ITEMS MARKED *ARE SARA TITLE III REPORTABLE |  |  |  |  |  |
| SECTION III - PHYSICAL DA TA |  |  |  |  |  |
| BOILING POINT°F | 210°F | pH (CONC.) | Less than 1 |  |  |
| SOLUBILITY IN WATER | Infinite | pH (USE DILUTION) | - |  |  |
| % VOLATILE BY WEIGHT | 62.0 | EVAPORATION RATE (WATER = 1) | 1.0 |  |  |
| SPECIFIC GRAVITY | 1.2 | LIQUIDJX] POWDER [ |  | 3 PASTE) j. AEROSOL [~~] |  |
| APPEARANCE AND ODOR | Clear Purple Liquid — Chemical Odor |  |  |  |  |
| SECTION IV -FIRE AND EXPLOSION DATA |  |  |  |  |  |
| FLASH POINT (TEST METHOD) . NONE (TAG C. C.) |  |  |  | FLAMMABLE LIMITS |  |
| EXTINGUISHING MEDIA [ ¦ OX, QJ R2AM QJ DRY CHEMICAL QJ WATER ["••[OTHER |  |  |  | UPPER LOWER |  |
|  |  |  |  | N/A N/A |  |
| SPECIAL FIRE FIGHTING PROCEDURES | None |  |  |  |  |
| UNUSUAL FIRE AND EXPLOSION HAZARDS | None |  |  |  |  |

/ — HE A L TH HAZA RD DATA

URE

Corrosive to eyes and skin

T-L-V-  1 mg/m3

UTES

Not Determined

iocedures      Rinse skin with water. Flush eyes 1.5 minutes. If irritation persists, see physician.   If injested, drink several large glasses of water. See physician.

V — RE A CT1VITY DA TA

STABLE

CONDITIONS TO AVOID

Alkali and metals

UNSTABLE

lity

Alkali and metals

riON PRODUCTS

Not determined

POLYMERIZATION

WILL OCCUR

WILL NOT OCCUR

// — SPILL OR LEAK PROCEDURES

3SAL

Pick up with mops to plastic bucket. Rinse area with alkaline detergent followed by clear water rinse.

Carefully neutralize with an alkali, dilute with large amounts of water and sewer

//// — SPECIAL PROTECTION INFORMATION

Goggles

ventilation     Normal to area

Rubber gloves

RESPIRATORY

protection     None

None in normal use

( — SPECIAL PRECAUTIONS

JSIN

sid storage    Keep from freezing

JS

KEEP OUT OF REACH OF CHILDREN

DB K.-

Chester E. Zackrison, Director of Research

.lability

ins or methods of use are beyond our control, we do not assume any responsibility and expressly disclaim any liability for any use of tho rnation contained herein is believed lo be true and accurate but all statements or suggestions are made without any warranty, express 01 linq.accuracy of Hie information, the hazards connected with ttio use of the material orjhe results to be obtained (torn the use thereof.